Communication with

VT200 inverter by

OpenCAN

V1.0

Website: http://www.we-con.com.cn/en

Phone: 86-591-8786886

Overview

This document describes the communication between Wecon HMI PI8070-A and VT200 inverter by OpenCAN,introduces it in three parts:

⚫  Configurations of Inverter

⚫  CAN command table

⚫ 	Software Configurations of HMI

I Configurations of Inverter

①Click[PRG],and choose the [P0]

Figure 1-1

②Choose[P0.02],and set it as [2]

Figure 1-2

Figure 1-3

③Choose[P0.03],and set it as [9]

Figure1-4

Figure1-5

④Choose[C7]

Figure 1-6

⑤Choose[C7.10],and set it as [1]

✎Note: Parameter C7.10 sets the station number of the device, which will affect the frame ID.

The data range is 1-15, and the default value is 1 if it is out of range.

Figure 1-7

Figure 1-8

⑤Choose[C7.11],and set it as [4]

✎Note:Parameter C7.11 sets the baud rate of the device.

| Value | 0 | 1 | 2 | 3 | 4 | 5 | other |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Baud Rate | None | 50K | 100K | 125K | 250K | 500K | 250K |

Figure 1-9

Figure 1-10

Ⅱ CAN command table

2.1 RPDO Control Message - 1 (Start and stop)

ID: 0 X 201(related to C7.10)

Example: The C7.10 be set as [1],the ID is set as [00 00 02 01]

The C7.10 be set as [2],the ID is set as [00 00 02 02]

Baud Rate:related to C7.11

Example: The C7.11 be set as [1],the Baud Rate is set as [50K]

The C7.11 be set as [4],the Baud Rate is set as [250K]

Frame format:Data Frame

Frame Type:Standard Frame

Length: 8 Byte

| Byte | Data format | Byte-name | Bit | Function |
| --- | --- | --- | --- | --- |
| Byte0 | One-Byte | Start Control | Bit0- Bit7 | Start: Bit 0.1=1&Bit 0.2=1 Stop：Bit0.2=0 Reset：Bit0.7=1 |
| Byte1 | One-Byte | Direction Control | Bit0- Bit7 | FWD：Bit 1.4=0 REV：Bit 1.4=1 |
| Byte2-3 | Little-endian | --- | Bit0- Bit15 | --- |
| Byte4-5 | Little-endian | --- | Bit0- Bit15 | --- |
| Byte6-7 | Little-endian | --- | Bit0- Bit15 | --- |

Example: ID:00 00 02 01

Message: 06 10 00 00 00 00 00 00

The inverter will start Reversal work.

2.2 RPDO Control Message - 2 (Frequency Control)

ID: 0 X 301(related to C7.10)

Example: The C7.10 be set as [1],the ID is set as [00 00 03 01]

The C7.10 be set as [2],the ID is set as [00 00 03 02]

Baud Rate:related to C7.11

Example: The C7.11 be set as [1],the Baud Rate is set as [50K]

The C7.11 be set as [4],the Baud Rate is set as [250K]

Frame format:Data Frame

Frame Type:Standard Frame

Length: 8 Byte

| Byte | Data format | Byte-name | Bit | Function |
| --- | --- | --- | --- | --- |
| Byte0-1 | Little-endian | Frequency Control | Bit0- Bit15 | 0-10000 Corresponding to 0-max frequency |
| Byte2-3 | Little-endian | --- | Bit0- Bit15 | --- |
| Byte4-5 | Little-endian | --- | Bit0- Bit15 | --- |
| Byte6-7 | Little-endian | --- | Bit0- Bit15 | --- |

Example: ID:00 00 03 01

Message: 13 88 00 00 00 00 00 00

The inverter Frequency will be set as half of the max frequency

(1388H = 5000,5000/10000 * 100%=50%)

Ⅲ Software Configurations of HMI

3.1 PIStudio

Figure3-1

3.2 New project

①[Home]→[New]→[New project]→Set the name,series and model of HMI→[OK]

✎Note:Please choose the device support CAN function.

Figure 3-2

3.3 Communication configuration

① [project]→[communication]→set the protocol and com parameters

Figure 3-3

② Choose protocol:

[setting]→[communication device]:

[communication]CAN, [Device type]CAN Device,[Protocol]Open CAN→[OK]

Figure 3-4

③Baud rate setting:

[COM port setting]:[Baud rate]250000

✎Note: Baud rate parameters and inverter parameters need to be set consistently, please set them according to theactual situation.

Figure 3-5

④OpenCAN setting:

[OpenCAN setting]→[Add]→Open the Data access Window

Figure 3-6

Take the [Start Stop] message as an example:

ID:00 00 02 01

Frame format:Data Frame

Frame Type:Standard Frame

Hex:00 00 00 00 00 00 00 00(8 Bytes)

Figure 3-7

Send Value from Frame NO. 1.1(the first bit) 16 Byte Length.

And send these value begin [CDATA_X_0.0]

Figure 3-8

After these setting,you can use the address to control the inverter.

| Start: Stop： Reset： | Bit 0.1=1&Bit 0.2=1 Bit0.2=0 Bit0.7=1 | CDATA_X_0.1=1 & CDATA_X_0.2=1 CDATA_X_0.2=0 CDATA_X_0.7=1 |
| --- | --- | --- |
| FWD： REV： | Bit 1.4=0 Bit 1.4=1 | CDATA_X_1.2=0 CDATA_X_1.2=1 |

Figure 3-9

More information about OpenCAN Setting,Please check the WIKI:

OpenCAN - Wecon (we-con.com.cn)

3.4 Project making

①Using PIStudio to program a HMI project depending on the requirements.

Figure3-10