Communication with

OMRON NX/NJ

series

V1.0

Website: http://www.we-con.com.cn/en

Phone: 86-591-8786886

Overview

This document describes the communication between Wecon HMI PI3070ig and OMRON NJ301-1100 via Ethernet port, and introduces it in three parts:

⚫  Software Configurations of PLC

⚫  Software Configurations of HMI

⚫ 	Cable Wiring

I Software Configurations of PLC

  - PLC programmer software

Figure 1-1

1.2 New PLC project

Open Sysmac Studio, create a new project or open the project that has been created.

Create new project:[New Project]→[Select Device]

Category:Controller;Device:NJ301-1100;Version:1.60→[Create]

Figure 1-2

1.3 Viewing/Configuring Ethernet Port Parameters

PLC Default Parameters:[Configurations and Setup]→[Bulit-in EtherNet/IP Post Settings]→[TCP/IP]→[IP Address]:Fixed setting :IP address 192.168.56.126

Figure 1-3

1.4 Create and Export the Freetags

①[Programming]→[Data]→[Global Variables]→Create the Freetags you need,notice the [Data type] and set the Network Publish as[Publish Only]

Figure 1-4

②[Tools]→[Export Global Variables]→[CX-Designer]

Figure 1-5

③Create a .CSV file and Paste the [CX-Designer] into it

Figure 1-7

Figure 1-8

1.5 Programming PLC project

Using Sysmac Studio to program a PLC project depending on the requirements.

Figure 1-9

1.6 PC and PLC connection

①[Controller]→[Communication Setup]→Setting [Connection type]→[OK]

Figure 1-10

1.7 Downloading PLC project

[Controller]→[Online]→[Transfer]→[To Controller]

Figure 1-11

II Software Configurations of HMI

2.1 PIStudio

Figure 2-1

2.2 New project

[Home]→[New]→[New project]→Set the name,series and model of HMI→[Next]→[Screen style] →[OK].

Figure 2-2

2.3 Communication configuration

① [project]→[communication]→set the protocol and com parameters

Figure 2-3

② Choose protocol:

select connection→[setting]→[communication device]:[communication]Ethernet, [Protocol]Ethernet, [Device type]OMRON,[Protocol]OMRON Ethernet/IP(NX/NJ Series)→[OK]

Figure 2-4

③COM port setting:

[device No.]1→[setting] →[PLC IP Address]:192.168.56.126→[OK].

✎Note: Serial port parameters and PLC parameters need to be set consistently, please set them according to theactual situation.

Figure 2-5

④Labels import:

[Labels manage]→[Import Tags From File]→Choose the .CSV file you need→[Open]→[Save]→[OK]

Figure 2-6

2.4 Project making

①Using PIStudio to program a HMI project depending on the requirements.

Figure 2-7

②After programming[Compile],you can download the project into HMI when compilation complete

Figure 2-8

2.5 Downloading project

[Home]→[Download]→[PC Port]→[PC to HMI]→[OK].

Figure 2-9

III Cable Wiring

3.1 Cable Wiring

Pin Definition Diagram：

Figure 3-1