V-Box OpenCloud Mode Connect to AWS IoT

- Log in AWS

Login aws account and click“Connect an IoT device”

- Create policy

- Click “Secure”--->“Policies”--->“Create policy”--->Click “Create”

- Name the policy--->Click “JSON”--->Copy the following content--->Click “Create”

- {

"Version": "2012-10-17",

"Statement": [

{

"Effect": "Allow",

"Action": [

"iot:Connect",

"iot:Publish",

"iot:Subscribe",

"iot:Receive",

"greengrass:Discover"

],

"Resource": "*"

}

]

}

- Create things

- Click “Manage”--->“Things”--->“Create things”--->“Create single thing”

Name the thing--->Click “Next”

Select the way to create certificate

Select policy

- Test with MQTT.fx tool

Click “View Setting” to get the “Broker Adress”

Create one connection in MQTT.fx tool, set broker port as 8883.

Upload the CA File, Client Certificate File, Client Key File

Publish message to topic “TEST”

Click”Test”--->”MQTT test client”--->”Subscrible to a topic”, to get message publish from MQTT.fx tool.

And we can also send message form AWS platform to MQTT.fx tool.

- Configurate in CloudTool

Copy the same setting in MQTT.fx to MQTT configuration

Add a lua script and copy the lua demo into it.

sprint = print

--Cloud mode interface to obtain the MQTT information configured by the cloud platform: (5 returns, namely the server address, client ID, connection table, last word table, certificate table)

local MQTT_URL, MQTT_CLIENTID, MQTT_CFG, MQTT_LWT, MQTT_CART = mqtt.setup_cfg()

--publish to topics

local pub_RE_TOPIC = string.format('TEST')

--Subscribe topics

local Subscribe_RE_TOPIC1 = string.format('TEST')

--variable

local last_time = 0

--Timing main function

function aws.main()

sprint(os.date("%Y-%m-%d %H:%M %S", os.time()) .. " aws.main start")

if g_mq then

if g_mq:isconnected() then

send_Data()

else

if os.time() - last_time > 5 then

last_time = os.time()

mymqtt_connect()

end

end

else

mymqtt_init()

end

sprint(os.date("%Y-%m-%d %H:%M %S", os.time()) .. " aws.main end")

end

-- Initialize MQTT

function mymqtt_init()

sprint(string.format("mqtt init mqtt_url:%s mqtt_clientid:%s", MQTT_URL, MQTT_CLIENTID))

g_mq, err = mqtt.create(MQTT_URL, MQTT_CLIENTID) -- Create the object and declare it as a global variable

if g_mq then

g_mq:on("message", mymqtt_msg_callback) -- Register to receive message callbacks

sprint("mqtt init success")

else

sprint("mqtt init failed:", err)

end

end

-- Connect to MQTT server

function mymqtt_connect()

sprint("mqtt connecting...")

local stat, err = g_mq:connect(MQTT_CFG,MQTT_LWT, MQTT_CART)

if stat == nil then

sprint("mqtt connect failed:", err)

return

else

sprint("mqtt connected")

end

g_mq:subscribe(TEST, 0)

end

-- Receive MQTT message callback function

function mymqtt_msg_callback(topic, msg)

print("topic:",topic)

print("revdata:",msg)

local revData = json.decode(msg)

print (revData)

if topic == Subscribe_RE_TOPIC1 then --Process topic information subscribed from the cloud

if string.match(topic,Subscribe_RE_TOPIC1) then

--if revData ~= nil then

for k,v in pairs (revData) do

print("printing revdata after kv here")

print (k,v)

end

print ("current state is",fanstate)

--end

end

end

end

--Get real-time data

function getData()

local jdata = {}

local addr = bns_get_alldata()

print(json.encode(addr))

for i,v in pairs(addr) do

if v[2] == 1 then

jdata[v[3]] = v[4]

end

end

return jdata

end

--send data

function send_Data()

local pub_data =

{

123

}

sprint(json.encode(pub_data))

print("..........",pub_RE_TOPIC)

return g_mq:publish(pub_RE_TOPIC, json.encode(pub_data), 0, 0)

end

Get message in AWS